MODULE 1

Q1. In which OSI layer the Wi-Fi standard/protocol fits?

Wi-Fi, which is defined by the IEEE 802.11 standard, primarily operates at two layers of the OSI (Open Systems Interconnection) model:

1. Physical Layer (Layer 1)

At this layer, Wi-Fi is responsible for transmitting raw data over radio waves. The key functions include:

- Modulation and Encoding – Converts digital data into radio signals for wireless transmission.

- Frequency Bands – Operates in 2.4 GHz, 5 GHz, and 6 GHz frequency ranges.

- Signal Strength and Interference Handling – Manages transmission power and interference from other wireless devices.

- Wireless Transmission Techniques – Uses technologies like OFDM (Orthogonal Frequency-Division Multiplexing) and DSSS (Direct Sequence Spread Spectrum) for reliable data transfer.

2. Data Link Layer (Layer 2)

Wi-Fi functions at the Data Link Layer, specifically in the MAC (Medium Access Control) sublayer. The key responsibilities here include:

- Media Access Control – Uses CSMA/CA (Carrier Sense Multiple Access with Collision Avoidance) to manage access to the shared wireless medium and prevent data collisions.

- Frame Formation and Transmission – Divides data into frames and ensures they are correctly sent and received.

- Error Detection and Retransmission – Implements ACK (Acknowledgment) frames and Automatic Repeat reQuest (ARQ) for error control.

- Security Mechanisms – Supports encryption and authentication methods like WPA2, WPA3, and WEP.

While Wi-Fi primarily operates at Layers 1 and 2, it also enables higher-layer protocols such as:

- IP (Internet Protocol) at Layer 3 – Facilitates routing over wireless networks.

- TCP/UDP (Transport Layer) at Layer 4 – Manages reliable/unreliable communication over Wi-Fi.

Q2. Can you share the Wi-Fi devices that you are using day to day life, share that device's wireless capability/properties after connecting to the network. Match your device to corresponding Wi-Fi Generations based on properties

Wi-Fi devices that I am using in my day-to-day life are laptop and smartphone.

| Device | Wireless Adapter/Chipset | Frequency Band | Wi-Fi Standard | Max Speed (Mbps) | Wi-Fi Generation |
| --- | --- | --- | --- | --- | --- |
| HP Envy x360 Laptop | Wi-Fi 6 | 2.4 GHz, 5 GHz | 802.11a/b/g/n/ac/ax | Up to 9.6 Gbps | Wi-Fi 6 |
| iPhone 17 Pro Max | Apple Wi-Fi 7 chipset | 2.4 GHz, 5 GHz, 6 GHz | 802.11a/b/g/n/ac/ax/be | Very high (multi Gbps) | Wi-Fi 7 |

Q3. What is BSS and ESS?

| Feature | BSS (Basic Service Set) | ESS (Extended Service Set) |
| --- | --- | --- |
| Definition | A single access point (AP) and its connected devices. | Multiple BSS units connected to form a larger network. |
| Number of APs | One AP | Multiple APs |
| Coverage Area | Small (Limited to the range of a single AP) | Large (Covers multiple APs' combined range) |
| Roaming Support | Not supported (devices stay within one AP range) | Supported (seamless transition between APs) |
| Communication | Devices communicate directly with a single AP. | Devices can switch between APs while staying connected. |
| Network Type | Used in home Wi-Fi, small office networks. | Used in large enterprises, universities, airports, etc. |
| Identifier | Identified by a BSSID (AP’s MAC address). | Uses multiple BSSIDs under a common SSID. |
| Example | A home Wi-Fi router with connected devices. | A campus Wi-Fi network with multiple APs providing continuous coverage. |

- BSS is a single access point and its connected devices, suitable for small networks.

- ESS is a group of multiple BSS units, providing larger coverage and seamless connectivity.

Q4. What are the basic functionalities of Wi-Fi Accesspoint?

A Wi-Fi Access Point (AP) is a networking device that allows wireless devices to connect to a wired network. It serves as a bridge between wired Ethernet networks and wireless clients, enabling seamless communication.

Basic Functionalities of a Wi-Fi Access Point

| Functionality | Description |
| --- | --- |
| Wireless Signal Transmission | Transmits and receives Wi-Fi signals, allowing wireless devices to connect to the network. |
| SSID Broadcasting | Advertises the Service Set Identifier (SSID), allowing users to discover and connect to the network. |
| Authentication & Security | Supports security protocols such as WPA2, WPA3, and 802.1X authentication to prevent unauthorized access. |
| Traffic Management | Manages data flow between connected devices and the wired network to optimize performance. |
| Multiple Device Connectivity | Supports multiple wireless clients simultaneously using MIMO (Multiple-Input Multiple-Output) technology. |
| Frequency Band Support | Operates on 2.4 GHz, 5 GHz, and 6 GHz bands depending on the Wi-Fi standard (Wi-Fi 4, 5, 6, 6E). |
| Roaming Support | In an Extended Service Set (ESS), allows devices to switch between APs without disconnecting. |
| QoS (Quality of Service) | Prioritizes certain types of traffic (e.g., video streaming, VoIP) for better performance. |
| Wired Network Integration | Connects to a router or switch via Ethernet to enable internet and network access. |
| Power over Ethernet (PoE) Support | Some APs can receive power through Ethernet cables, eliminating the need for a separate power source. |
| Guest Network Creation | Allows setting up a separate network for guests to improve security and limit access. |
| Mesh Networking | Some APs support mesh Wi-Fi, allowing multiple APs to extend network coverage seamlessly. |

Q5. Difference between Bridge mode and Repeater mode

Both Bridge Mode and Repeater Mode are used to extend network connectivity, but they function differently.

Key Differences:

| Feature | Bridge Mode | Repeater Mode |
| --- | --- | --- |
| Purpose | Connects two separate networks. | Extends the range of an existing Wi-Fi network. |
| Operation | Works at Layer 2 (Data Link Layer). | Works at Layer 1 (Physical Layer). |
| Network Type | Connects two different networks. | Extends one single network. |
| IP Addressing | Networks retain their own IP ranges. | All devices stay in the same IP range. |
| Performance | No signal loss; provides direct network bridging. | May reduce speed due to retransmission delays. |
| Usage | Used in enterprises, industrial setups. | Used in homes, small offices to eliminate Wi-Fi dead zones. |
| Example | Connecting two office networks in different buildings. | Using a repeater to boost Wi-Fi in a large home. |

- Bridge Mode is for connecting two separate networks efficiently.

- Repeater Mode is for extending Wi-Fi coverage by amplifying and retransmitting signals.

Q6. What are the differences between 802.11a and 802.11b?

| Feature | 802.11a | 802.11b |
| --- | --- | --- |
| Release Year | 1999 | 1999 |
| Frequency Band | 5 GHz | 2.4 GHz |
| Max Data Rate | Up to 54 Mbps | Up to 11 Mbps |
| Channel Bandwidth | 20 MHz | 22 MHz |
| Modulation | OFDM (Orthogonal Frequency Division Multiplexing) | DSSS (Direct Sequence Spread Spectrum) |
| Range | Shorter due to high frequency | Longer due to lower frequency |
| Interference | Less interference, as fewer devices use 5 GHz | More interference due to congestion in 2.4 GHz |
| Wall Penetration | Poorer (struggles with obstacles) | Better (penetrates walls more effectively) |
| Power Consumption | Higher due to complex modulation | Lower, making it more energy-efficient |
| Network Type | Used mainly in enterprise and business environments | Used widely for residential and consumer devices |
| Adoption | Less popular due to high cost and short range | Widely adopted for early Wi-Fi networks |
| Compatibility | Not compatible with 802.11b/g | Backward compatible with 802.11g |
| Use Cases | Business, high-speed applications | Home networks, IoT, and low-speed internet |

Q7. Configure your modem/hotspot to operate only in 2.4Ghz and connect your laptop/Wi-Fi device , and capture the capability/properties in your Wi-Fi device. Repeat the same in 5Ghz and tabulate all the differences you observed during this

Connect the laptop to 2.4 Ghz and run netsh wlan show interfaces in the cmd. Repeat the same after connecting it to 5 Ghz. The differences observed during the execution of commands are as follows:

| Feature | 2.4 GHz | 5 GHz |
| --- | --- | --- |
| Radio Type | 802.11n | 802.11ac |
| Band | 2.4 GHz | 5 GHz |
| Channel | 1 | 40 |
| Recieve Rate (Mbps) | 144.4 | 526.5 |
| Transmit Rate (Mbps) | 130 | 180 |

Q8: What is the Difference Between IEEE and WFA?

1. IEEE (Institute of Electrical and Electronics Engineers)

- Definition:

  - A professional organization that develops global technical standards for various industries, including networking and telecommunications.

- Role in Wi-Fi:

  - Defines the 802.11 standards (Wi-Fi protocols) such as 802.11a, 802.11b, 802.11n, 802.11ax (Wi-Fi 6).

  - Focuses on technical specifications, protocols, and interoperability.

- Example Contribution:

  - IEEE 802.3 (Ethernet), IEEE 802.11 (Wi-Fi), IEEE 802.1x (Authentication protocol).

2. WFA (Wi-Fi Alliance)

- Definition:

  - A non-profit industry consortium that certifies Wi-Fi products for compatibility, performance, and security.

- Role in Wi-Fi:

  - Ensures devices from different manufacturers work together seamlessly.

  - Develops Wi-Fi branding (e.g., Wi-Fi 4, Wi-Fi 5, Wi-Fi 6).

  - Introduces security and performance enhancements (e.g., WPA2, WPA3).

- Example Contribution:

  - Wi-Fi CERTIFIED™ program, Wi-Fi Direct, Passpoint, WPA3 security standard.

Key Differences:

| Feature | IEEE (Institute of Electrical and Electronics Engineers) | WFA (Wi-Fi Alliance) |
| --- | --- | --- |
| Purpose | Develops technical standards for networking and communication. | Certifies Wi-Fi products for interoperability. |
| Focus Area | Defines Wi-Fi protocols (802.11 standards). | Ensures Wi-Fi devices are compatible and secure. |
| Membership | Open to researchers, engineers, and industry professionals. | Includes companies like Intel, Cisco, Qualcomm, etc. |
| Wi-Fi Involvement | Creates the 802.11 Wi-Fi standards. | Certifies Wi-Fi products and introduces Wi-Fi branding. |
| Security Role | Defines authentication mechanisms like 802.1x. | Develops Wi-Fi security protocols (e.g., WPA2, WPA3). |
| Branding | No branding – just technical standards. | Creates Wi-Fi branding (Wi-Fi 4, 5, 6, etc.). |
| Example Contribution | 802.11ax (Wi-Fi 6) standard definition. | Wi-Fi 6 certification program, WPA3 encryption. |

Q9. List down the type of Wi-Fi internet connectivity backhaul, share your home/college's wireless internet connectivity backhaul name and its properties

A backhaul is the network connection that links the Wi-Fi access points (APs) to the main internet service provider (ISP) or core network. The backhaul can be wired (fiber, Ethernet) or wireless (Wi-Fi, satellite, cellular).

Types of Wi-Fi Backhaul Connections

| Backhaul Type | Description |
| --- | --- |
| Fiber Optic Backhaul | Uses fiber-optic cables for high-speed, low-latency internet connectivity. Common in enterprise networks and 5G backhaul. |
| Ethernet Backhaul | Connects Wi-Fi access points via wired Ethernet cables for stable, high-speed connections. Used in homes, offices, and businesses. |
| Wireless Backhaul (PtP/PtMP) | Uses microwave, millimeter-wave, or Wi-Fi links to connect APs without physical cables. Used in rural and large outdoor deployments. |
| Cellular (4G/5G) Backhaul | Uses LTE/5G networks to provide Wi-Fi connectivity where wired connections are unavailable. Common in remote areas and mobile hotspots. |
| Satellite Backhaul | Provides internet via satellites, useful for rural and remote locations with no wired infrastructure. Higher latency compared to fiber. |
| Power Line Communication (PLC) Backhaul | Uses electrical power lines for network data transmission. Less common but useful in industrial environments. |

College Wi-Fi Backhaul

Backhaul Type: Ethernet
Provider: BSNL
Speed: 100-500 Mbps
Latency: 5-10 ms

Q10. List down the Wi-Fi topologies and use cases of each one.

Wi-Fi topology refers to the network structure and how devices connect to each other and the internet.

Types of Wi-Fi Topologies and Their Use Cases

| Wi-Fi Topology | Description | Use Cases |
| --- | --- | --- |
| Infrastructure Mode | Devices connect to a central Access Point (AP), which manages communication and connects to the internet. | - Home and office Wi-Fi networks. - Public Wi-Fi (cafes, airports). |
| Ad-hoc Mode (Peer-to-Peer) | Devices communicate directly with each other without an access point. | - Temporary networks (file sharing, gaming). - Device-to-device communication. |
| Mesh Wi-Fi | Multiple access points (APs) work together to create a seamless network, improving coverage and reliability. | - Large homes and offices. - Smart city and IoT deployments. - Campus-wide Wi-Fi. |
| Workgroup Bridge Mode | Workgroup Bridge Mode allows a Wi-Fi device to bridge its wired clients to a remote network or access point wirelessly, useful for connecting distant wired devices. | Commonly used in scenarios where running Ethernet cables is impractical, like point-to-point links between buildings. |
| Hotspot (Tethering) | A mobile device or router shares its internet connection over Wi-Fi. | - Mobile hotspots. - Temporary Wi-Fi access for travel. |
| Bridge Mode | Bridge Mode involves connecting two separate network segments or LANs wirelessly. | - Expanding Wi-Fi in large buildings. - Campus and warehouse networks. |
| Extender/Repeater Mode | Wi-Fi extenders boost existing signals to reach dead zones. | - Homes with weak Wi-Fi spots. - Extending Wi-Fi in offices. |