MODULE 5

1. Key Features of Wi-Fi 6, 6E, and 7 Compared to Wi-Fi 5 (802.11ac)

Wi-Fi 5 (802.11ac):

- Operates only in 5 GHz band.

- Supports up to 8 spatial streams.

- Introduced MU-MIMO (downlink only).

- Max throughput: ~3.5 Gbps.

Wi-Fi 6 (802.11ax):

- Operates in 2.4 GHz and 5 GHz.

- Introduces OFDMA and uplink MU-MIMO.

- Improved efficiency and reduced latency.

- Max theoretical speed: ~9.6 Gbps.

- TWT for power saving.

Wi-Fi 6E:

- Extension of Wi-Fi 6 into 6 GHz band.

- Offers up to 1.2 GHz of additional spectrum.

- Less interference, higher capacity.

- Supports 160 MHz and 320 MHz channels.

Wi-Fi 7 (802.11be):

- Operates in 2.4 GHz, 5 GHz, and 6 GHz.

- Supports up to 320 MHz bandwidth.

- Multi-Link Operation (MLO).

- 4096-QAM modulation (higher data rates).

- Extremely low latency for AR/VR and gaming.

2. Role of OFDMA in Wi-Fi 6 and Network Efficiency

- OFDMA (Orthogonal Frequency Division Multiple Access) subdivides a channel into smaller subchannels (Resource Units).

- Multiple users can transmit simultaneously.

- Reduces latency and increases spectrum efficiency.

- Improves performance in dense environments like stadiums and offices.

3. Benefits of Target Wake Time (TWT) in Wi-Fi 6 for IoT Devices

- Schedules specific wake and sleep times for devices.

- Reduces power consumption significantly.

- Extends battery life of IoT sensors and wearables.

- Decreases contention and improves reliability in dense networks.

4. Significance of the 6 GHz Frequency Band in Wi-Fi 6E

- Provides additional unlicensed spectrum (up to 1.2 GHz).

- Supports more non-overlapping channels (e.g., fourteen 80 MHz or seven 160 MHz channels).

- Less crowded, leading to lower interference.

- Ideal for bandwidth-intensive applications.

5. Wi-Fi 6 vs Wi-Fi 6E: Range, Bandwidth, Interference

| Feature | Wi-Fi 6 | Wi-Fi 6E |
| --- | --- | --- |
| sFrequency | 2.4 GHz & 5 GHz | 6 GHz (plus 2.4 & 5 GHz) |
| Range | Better at 2.4 GHz | Slightly lower at 6 GHz |
| Bandwidth | Up to 160 MHz | Up to 320 MHz |
| Interference | More crowded bands | Cleaner, less congested |

6. Major Innovations in Wi-Fi 7 (802.11be)

- Multi-Link Operation (MLO): Parallel links on different bands.

- 320 MHz channel bandwidth: Higher throughput.

- 4096-QAM: Higher data encoding efficiency.

- Preamble Puncturing: Better channel utilization.

- Deterministic latency: Ideal for AR/VR, gaming, and industrial IoT.

7. Multi-Link Operation (MLO) and Its Impact

- Enables devices to use multiple channels across bands (2.4, 5, 6 GHz) simultaneously.

- Impact:

  - Increased throughput.

  - Improved reliability through link aggregation.

  - Lower latency by selecting the best path dynamically.

8. Purpose of 802.11k and 802.11v in Roaming

- 802.11k: Neighbor Report Protocol.

  - Provides clients with info about nearby APs.

  - Helps devices make informed roaming decisions.

- 802.11v: Network-assisted roaming.

  - Enables APs to suggest better APs for clients to move to.

  - Also helps with battery savings by reducing scanning.

9. Fast BSS Transition (802.11r)

- Allows faster roaming by pre-authenticating with target AP.

- Reduces handoff time to <50 ms.

- Beneficial for voice/video applications and mobile users.

10. How 802.11k/v/r Work Together for Seamless Roaming

- 802.11k provides a list of nearby APs.

- 802.11v guides client to a better AP.

- 802.11r enables fast authentication.

Together:

- Minimize connection delays.

- Reduce packet loss.

- Enhance user experience in enterprise and high-density networks.